Young Inventor’s Guide to Nuclear Reactor Design

# H. Peter Alesso

Young Inventor’s Guide to Nuclear Reactor Design

Also by H. Peter Alesso

Self-Improving AI

AI Operating Systems

Absolute Zero Reasoner

On the Cusp of SuperIntelligence

AI Predictive Modeling

AI Game Theory

Puzzling AI Trends 2025-26

AI Quantum Field Theory

AI Langlands Program

Tomorrow's Film Studio Today

Vibe Coding by Example

Connections: Patterns of Discovery

Thinking on the Web

Developing Semantic Web Services

The Intelligent Wireless Web

e-Video

Young Inventor’s Guide to Nuclear Reactor Design

H. Peter Alesso © 2026

hpeteralesso.com

AI HIVE Publications

Pleasanton, CA 94566

Edition 1.00

# Synopsis

# REACTOR DESIGN

1.0 Overview

Nuclear fission is the splitting of the nucleus into two or more fission fragments with one to three neutrons and a great deal of energy (about 200 Million electron-volts).  Fission can occur with high probability for fissile materials such as Uranium-235 or Plutonium-239.  Those neutrons emitted immediately from fission are called prompt neutrons.  While those neutrons released 0.2 to 55 seconds after fissioning has occurred are called delayed neutrons.  When the number of neutrons being produced from all sources is exactly equal to the number of neutrons being eliminated by all mechanisms then rate of change in the neutron population is zero.  At that time, the neutron population is balanced and the reactor is called CRITICAL.  If the neutron population is growing the reactor is called SUPERCRITICAL and if the population is declining the reactor is called SUBCRITICAL.

The main goal of reactor physics is to compute the free neutron population throughout the reactor.  With this, the nuclear reactions that  occur that effect reactor criticality can be computed.  The high energy (fast) neutron population can cause high energy fission reactions (fast fission) to occur, though with a relatively low probability.  Fast neutrons can also leak out of the reactor and be lost from further reaction.  Or they can be absorbed by materials that do not produced a fission.

Those fast neutrons that do not cause fast fission, get absorbed, or leak out and become lost, will be slowed down as they lose energy through collisions with the atoms of the surrounding materials.  Low energy (thermal) neutrons have a much higher probability of causing a fission in fuel than fast neutron.  Therefore, once the neutrons have slowed down adequately (again without being absorbed or lost through leakage) they can control the fissioning rate in the reactor and the thermal power production.

Controlling the reactor’s criticality and subsequent power generation, depends on controlling reaction rates through reactivity changes.  Reactivity is defined as a measure of criticality fluctuation.  Reactivity in a reactor can vary due to the coolant temperature, coolant pressure, control rod (deliberately stationed neutron absorbers) position, burnable poisons (deliberately placed materials that absorb neutrons in special regions), fuel depletion, and fission product poisoning (waste materials that absorb neutrons).

The choice of fuel will determine the probability of fission.  The choice of moderator will determine the probability that neutrons will slowdown without being absorbed or lost to leakage.  The choice of geometry and dimensions will play a large role in determining the rate at which neutrons are lost to leakage.  And the choice of coolant will also play a role in neutron absorption in the reactor.  Coolant will of course determine the rate of heat transfer from the reactor to the power conversion system as well as the overall system efficiency.

The design of a thermal nuclear reactor and aspects of its power conversion system is a complex integration of physics and engineering.  It requires aspects of neutron physics, thermal-hydraulics, stability and control, materials, structures, radiation shielding and more.  The choice of a reactor concept is based on extensive parametric studies that vary the selection of such fundamental variables as fuel, moderator, and stability, and power conversion systems.  In parametric evaluations, an attempt is made to estimate the gross effects that varying fundamental variables have in meeting the design objectives.

The first step in “DESIGN YOUR OWN REACTOR” is the selection of fissile fuel.  Only three nuclides, Uranium-233, Uranium-235 and Plutonium-239 are; (1) stable for storage, (2) “available”, (3) capable of fissioning by neutrons of all energies, from thermal (0.025 electron volts or less) to millions of electron volts.  Of these, only U-235 occurs in nature.  The others are produced artificially by neutron capture followed by radioactive decay.  Several other species capable of undergoing fission by neutrons of all energies, but most are highly radioactive and decay rapidly.  However, Plutonium-241, Americium-242, and Curium-245 are included as selection options.  While their stability is limited and they will be unavailable for decades to come, they are included because of the insights they provide when comparing parametric studies.  Similarly, an “Add Your Own Fuel “option is included which will allow you to input values for special materials for which you have your own data.

In order to minimized the required critical mass of fuel, it is desirable to have most of the fissions occur as a result of absorption of thermal neutrons.  This is achieved by distributing the fuel within a moderating material.  The function of the moderating material is to slow down the high-energy neutrons liberated in the fission reaction, mainly as a result of elastic scattering reactions.  The best moderators are materials consisting of elements of low mass number with little tendency to capture neutrons; examples are ordinary water, heavy water (deuterium oxide), beryllium, and carbon.  The relative amount and nature of the fuel and moderator determine the energies of most of the neutrons causing fission.  The relative amounts are often expressed as the Moderator-to-Fuel ratio (Nm/f) which is the ratio of an atom of moderator to an atom of fuel.

The critical Buckling B^2 for thermal unreflected nuclear reactors can be used to evaluate the critical size for a prescribed shape.

For a given material composition, the critical volume (and mass) will be minimal for a sphere.  This is because the sphere has the minimum area-to-volume ratio and neutron production takes place throughout the volume but neutron leakage out of the reactor occurs only at the surface.

Once reactor fuel and moderator have been selected and the geometry designated, it is possible to calculate the critical mass of reactor fuel from its geometric “Buckling” and subsequent critical moderator-to-fuel ratio.  In “DESIGN YOUR OWN NUCLEAR REACTOR”, this calculation is performed from Fermi age, thermal group, and homogeneous reactor approximations.  As a consequence, the results are only rough estimates and not as accurate as studies that use Monte Carlo simulations to obtain critical mass estimates for heterogeneous designs.  Once the critical mass of fuel is found, estimates of power generation, power density, heat flux, specific power, and thermal neutron flux.

The heat generated in the reactor core due to fission is removed by coolant such as, water, liquid metals, gases, and certain organic compounds.  To convert this heat to electrical energy, it must be either; (1) transferred from the coolant to a working fluid to produce steam (or by a hot gas) in a CLOSED LOOP power conversion system, or (2) directly run a turbine in an OPEN LOOP power conversion system.  The higher the temperature of the steam or working fluid, the greater the efficiency for conversion into useful power.  Hence, the desire to operate at the highest possible temperature.  Heat must be removed from the core at a rate that permits the coolant to attain a high temperature without causing thermal stresses and fuel center-line-temperatures which will damage the reactor fuel or structure.

“DESIGN YOUR OWN NUCLEAR REACTOR” has been composed to advantageously combine various criticality/thermal-hydraulic/control/ and material limitations into a self-consistent design, while avoiding violating physics and engineering principles.  As you gain experience by running many of your own designs you will begin to appreciate various trade-offs that must be made.  You will then reach a better understanding of the limitations of this program.

2.0  Nuclear Physics

The main goal of reactor physics is to compute, for any time “t”, the characteristics of the free-neutron population throughout the reactor.  With this, rates at which nuclear reactions occur can be predicted and thus how much nuclear power will be generated.  Nuclear fission is the splitting of the nucleus into two fission fragments:  one to three free neutron, and numerous Gamma (G) rays.  This can occur with high probability for U-233, U-235, Pu-239, or Pu-24l.  The fraction of the neutrons emitted between the energies E and E + dE is symbolized by C (E), which is the fission spectrum.  Not all the neutrons are emitted immediately - in an average fission the fraction b (0.65% for U-235) are emitted between 0.2 seconds and 55 seconds later.

Reactor control of pressure water reactors consists of controlling the reactivity changes in the core.  Reactivity is defined as the degree of criticality of a reactor (i.e., r = 1 - 1/Keff).  The reactivity insertion mechanisms are as follows:

1)	Reactor Coolant Temperature

2)	Reactor Pressure

3)	Control Rod Movement

4)	Fuel Depletion. (Age Effect Burnable Poison Depletion)

5)	Fission Product Poison Concentration.

rnet = rtemp + rpress + rrod + rage + rfpp

The change in the average temperature of the reactor coolant results in a change in moderator density.  This changes the rate at which neutrons slow down, and thus the neutron population.  Temperature changes also change the Doppler Broadening Effect - the width of the absorption cross sections in the resonance region.  Thus, it changes the neutron population through the amount of absorption of neutrons.  The composite of these two effects is (aT), the temperature coefficient, where if a reactor has a positive aT, an increase in temperature would cause a positive amount of reactivity to be inserted into the core.  Thus, there would be a positive feedback effect.  All reactors are required to have negative temperature coefficients in order to have a negative feedback effect and thus be self-regulating.

The pressure coefficient of reactivity also changes reactivity since an increase in pressure increases moderator density.  Changes in pressure are usually neglected since are positioned in the core to Control rods are neutron absorbers that effect reactivity.  By positioning more absorbent material near neutrons, negative reactivity can be added.

Differential rod worth is the change in reactivity due to the change in height of a control rod relative to the flux density.

Fuel and burnable poisons are depleted with time, and thus they affect reactivity in the core.

Some fission products are very strong absorbers of thermal neutrons (e.g., xenon), and so influence neutron population and thus reactivity.

At each period in the core lifetime, the core must be able to be critical, shutdown, or controlled over the extremes of possible core conditions with a certain margin for safety.

The purpose of the control rods is to start up the reactor, control average temperature of the coolant, shut down the reactor, and compensate for changes in core properties in order to maintain criticality.  The reactor operator controls Keff by rod motion.  Keff is controllable by human actions only because of delayed neutrons.  Prompt neutrons appear 10-4 seconds after a fission, while delayed neutrons appear 0.2 to 55 seconds after fission.

Reactor power is given by P = Po   where 1 is equal to mean generation time.  If neutrons were all prompt, power could change by a factor of 8,000 in 0.33 seconds for a change of Keff of 0.003.

3.0 ENGINEERING

Designing a nuclear reactor system is a complex project.  A nuclear plant includes a number of components similar to non-nuclear facilities such as heat exchangers, turbo-generators, engines, reduction gears, et cetera.  The design of a nuclear reactor system follows several phases.  First, objectives are defined and criteria are established.  Second, a preliminary design is prepared.  Third, the principal features of the reactor and its associated plant are defined in enough detail to permit cost estimation.

Design objectives might include desired power capacity of the plant.  The nature of fuel, moderator, coolant, and general configuration must be planned.  For a pressurized water reactor, the fuel must be enriched and fuel element geometries may be enriched and fuel element geometries may be cylindrical rods or flat plates.  The optimum degree of enrichment depends upon:  1) size of core, 2) thermal power required, 3) moderator to fuel ration, 4) diameter of fuel elements, and 5) fuel-cycle costs.

To reduce irradiation damage the choice of fuel can be a form of UO2, clad in stainless steel or zirconium alloy.  UO2 has the disadvantage of a relatively low heat conduction.  The diameter of the rod affects heat transfer, power density, fabrication costs, and resonance escape probability, and thus the conversion of U-238 to Pu-239.  Another factor in choice of rod diameter is the “cladding”.  Cladding depends upon chemical reactivity, neutron capture cross section, and thickness to provide support and cost.  Fission product gas pressure and corrosion will increase with time and also affect the “clad” thickness.

The final core development is a compromise between thermal and nuclear parameters.  In planning operating conditions, the maximum heat flux is calculated.  In order to allow for local deviations from average behavior, “Hot Channel” factors reduce maximum flux.

The fuel enrichment must yield an effective multiplication factor in a “cold, clean” condition to account for a decrease in reactivity upon heating of the core to the operating condition, the decrease during core life due to “burn-up” of fuel, the Doppler Effect, the accumulation of fission product poisons, the control rod absorption, and the burnable poisons (those which are added to extend core life).

The Heat-Removal Rate from the fuel to the coolant is a critical problem and usually limits the core design.  Certain limitations are imposed by such factors as temperatures, thermal stresses, corrosion, and erosion.  There are definite limits set upon the temperature of the interior of fuel elements, “cladding” temperature, and coolant temperature.  Limitations on flow rate of coolant and drop in pressure through the reactor have an important effect upon heat removal.

A thermal designer must ensure that all channels are safe with respect to thermal limits.  This can be accomplished by making sure that the most limiting channel is safe.  The “Hot Channel” is assumed to be located in the hottest region of the core and to have the most adverse (within tolerances) dimensions to thermal performance.  Thermal limits are set as: 1) no flow oscillations, 2) no instabilities, 3) no departure from nucleate boiling, and 4) no temperature greater than maximum peak centerline temperature allowable (for steady-state limit or for transient limit).

For a given channel at steady-state, all energy generated in the centerline of the rod must enter the channel.  Thus, the enthalpy and temperature of the coolant increases as it flows up the channel.  DNB is the departure from nucleate boiling caused by vapor coating of the area, which therefore does not allow heat flow and thus causes very rapid increase in surface temperature.

By using a simple energy balance on the fuel rod and employing the general energy equation, the relationship between pertinent parameters is obtained.  This is used to evaluate the combination of parameters as Tbulk, Tcenterline, Twall, m (mass flow rate), h (enthalpy), and X (quality of Steam).  Then these parameters are evaluated to see if their combination of values will violate a thermal limit.

A shielding system around the reactor is necessary to reduce radiation to tolerable levels.  The shield must be capable of absorbing neutrons and Gamma rays.  It is made of two components:  a thermal shield, and a biological shield.

4.0  REACTOR PROTECTION

1)  Automatic Protection:  Many transients occurring in the reactor are too rapid for human diagnosis and reaction.  Therefore, many automatic features have been incorporated into the operating circuitry.  A reactor “scram”, a rapid ( 1 seconds) insertion of control rods, and a “cut back” - a fast driving in the control rods, are safety features that may be initiated manually or automatically when certain limits are reaches.  For reactivite  excursions due to uncontrolled rod withdrawal, too rapid withdrawal of rods, cold water accident, or loss of coolant, protection is provided by a high “Start-Up” Rate (SUR) “scram” or by an overpower “scram.”

In case of loss of control due to control system failure, operator error, or a stuck rod, protection is provided by a high-temperature outlet “scram” or by an overpower “scram.”

For a power to flow mismatch due to either instrument error or control system malfunction, protection is provided by high-temperature outlet “scram.”

In case of a coolant failure due to a loss of flow, a loss of coolant, or a reactor vessel failure, protection is provided by loss of flow “scram,” a high outlet “scram” or a low pressure “scram.”

2)  Other Forms of Protection:  Interlocks are designed into reactor instrumentation in order to prevent operators from committing serious errors.  For example, the cold water interlocks prevent starting flow from a “cold” loop into the reactor.  This would cause colder water to add a great deal of reactivity into the reactor due to the negative temperature coefficient.

Personnel must be highly trained to operate a reactor safely during “Shut-Down,” during operations, or in emergency operations.  Procedures must be explicitly defined in manuals and rigidly adhered to by personnel.

5.0  RADIOACTIVE HAZARD

Nuclear plants face the problem of being restricted due to the possibility of radiological contamination.

The biological shields around the reactor in the form of the primary and secondary shields provide sufficient radioactive protection to allow workers access to all portions of the plant (except the reactor building during operations) for specified amounts of time.  The radiation leakage at normal operating power is less than the normal radiation dose present in the atmosphere.  Direct radiation is therefore not a hazard.

There are circumstances when primary coolant must be discharged from the primary loop, but these do not require large amounts of discharge and they may be contained in the tanks.

The radioactivity in the primary coolant comes from three sources.  First, Gamma rays from the fuel elements activate the primary coolant (water) yielding oxygen and nitrogen isotopes that decay with short half-lives.  Second, erosion of metallic elements that are in contact with the primary coolant provides a mobile source of materials that pass through the reactor and thus become activated.  The most notable of these are Cobalt, Iron, and Magnesium.  The Cobalt has a long half-life and is a source of the long term radioactive hazard that primary coolant environment.  The third source is fission products that recoil from fission and travel to the coolant.  The “cladding” also has impurities that can be activated and travel into the coolant.

Radioactivity from the reactor directly is in the form of Beta, Gamma, and neutron radiation.  Shielding of lead and stainless steel are employed to stop the Beta and Gamma radiation while the moderator (water) acts as a fast neutron remover and prevents any dangerous radiation from leaving the Reactor Compartment.

Power Transients and Their Effect

6.0  THE PROPULSION PLANT OF NUCLEAR SHIPS (an example)

As an example of operations consider a nuclear powered ship’s reactor. With the given initial conditions, the problem of an ordinary power transient will be followed throughout the Propulsion Plant.  Initial conditions are:

1)	Reactor critical, operating at power.

2)	Reactor self-sustaining.

3)	Ship at “All Stop.”

4)	Reactor Power = Low.

5)	TAV (average coolant temperature) = Normal.

6)	DT (Thot - Tcold) = Small.

7)	m (mass flow rate of primary coolant) - slow.

8)	Pressurizer level = Normal.

9)	Steam generator pressure = Normal.

10)	Turbine generators supplying electrical loads.

11)	Main engine condensers at vacuum.

12)	Constant TAV system (assumed).

The need for moving the ship imposes a demand for power.  The initial pressure in the steam generators is about 600 psia.  When the throttle is opened, steam flows through the main engineers to the main condensers which are at an initial vacuum.  The large pressure differential starts a mass flow rate of steam from the steam generators to the main condensers.

The steam generator is basically a tube and shell heat exchanger which maintains steam at saturation temperature and pressure.  Thus, when the mass flow rate of steam to the main condensers increases, it reduces the partial pressure of steam inside the steam generator and allows water to “flash” to steam, and the temperature of the steam generator water is reduced.  The partial pressure of steam drop, a control device, signals the steam generator water level control system to increase make-up feed water to keep an approximately constant level of water in the steam generator.  This make-up feed water is colder than the water already present, and thus it adds to a reduction of the temperature of the steam generator.

As the average temperature of this water declines, the temperature gradient between the primary coolant and the steam generator “secondary” water increases, thereby promoting between heat transfer.  The lower average steam generator temperature draws more heat from the primary coolant and reduces Tcold (the exit temperature of the primary coolant from the team generator).  This is obvious from the following equations:

(1)  Qp (heat rate primary) =

QSG (heat rate secondary)

Tsat is the first to drop.  Thus, (Tav - Tsat) increases, resulting, in Tcold decreasing initially to maintain balance.

TCold has been reduced several degrees.  It returns to the reactor where through the negative temperature coefficient it increases reactor reactivity.  A positive “Start-Up” Rate for power will now increase reactor power exponentially.  However, the rising power will raise the temperature of the coolant, and through the negative temperature coefficient, it will begin to cause negative reactivity.  This will stop the power rise and eventually, after several oscillations, power will reach a new steady-state value which will be higher than its initial value (See Figure).

If power doubled from its initial level, it follows (Thot - Tcold) must double (in this case, from A°F to 2A°F).  TAV would initially decrease as Tcold decreased, but as reactor power increases, thereby also increasing Thot, TAV will return to its initial level.

Primary pressure and pressurizer level will vary during the transient, but the resultant levels depend greatly upon the physical contraction of the pressurizer piping and will thus vary from system to system.

Meanwhile, steam from the steam generator is flowing to the main engines which in turn drive the ship through the normal propulsion train sequence.

The used steam is converted to water in the main condenser following normal marine engineering practice, and is pumped back to the steam generator.

7.0 CONCLUSION

The design  of a  THERMAL NUCLEAR REACTOR  and aspects  of its power  conversion  system  is  a  complex  integration  of  physics  and engineering.     It    requires    knowledge   of    neutron    physics, thermal-hydraulics, materials,  structures, and  more.  The choice  of a reactor concept is  based on extensive parametric studies  that vary the selection of such fundamental  variables as fuel, moderator, geometrical configuration,  coolant, and  power  conversion  system.  In  parametric evaluations,  an attempt  is made  to  estimate the  gross effects  that varying  fundamental   characteristics  have   in  meeting   the  design objectives.

"DESIGN  YOUR OWN  NUCLEAR  REACTOR"  is a  menu  driven program  with opportunity to input  certain parameters.  It allows the  user to select from  a   wide  variety   of  fundamental  materials,   geometries,  and configurations in order  to develope his own  nuclear reactor conceptual design.  Then,  important reactor variables  are calculated such  as the critical fuel  mass.  The results  are presented in both  summary table and graphical displays.  The program allows multiple iterations in order to accumulate cases  of individual reactor design for  comparison.  As a result, the  user can optimize  toward a specific  design characteristic and review the results.

This program will  prove beneficial both as an  educational device for college level  science and engineering  students and as  a prelimininary conceptual design tool for scientists and engineers.

Formulars for Buckling

Geometry	Buckling		Min. Critical

Volume

Spherical		(pi/r)^2		(130/B)^3

Cylinder		(2.405/R)^2 +	(161/B)^3

Parallelepiped	(pi/x)^2 +	(148//B)^3

(pi/y)^2 +

(pi/z)^2

where pi = 3.141526 for H = 1.847R

PREFACE

The design  of a  THERMAL NUCLEAR REACTOR  and aspects  of its power  conversion  system  is  a  complex  integration  of  physics  and engineering.     It    requires    knowledge   of    neutron    physics, thermal-hydraulics, materials,  structures, and  more.  The choice  of a reactor concept is  based on extensive parametric studies  that vary the selection of such fundamental  variables as fuel, moderator, geometrical configuration,  coolant, and  power  conversion  system.  In  parametric evaluations,  an attempt  is made  to  estimate the  gross effects  that varying  fundamental   characteristics  have   in  meeting   the  design objectives.

"DESIGN  YOUR OWN  NUCLEAR  REACTOR"  is a  menu  driven program  with opportunity to input  certain parameters.  It allows the  user to select from  a   wide  variety   of  fundamental  materials,   geometries,  and configurations in order  to develope his own  nuclear reactor conceptual design.  Then,  important reactor variables  are calculated such  as the critical fuel  mass.  The results  are presented in both  summary table and graphical displays.  The program allows multiple iterations in order to accumulate cases  of individual reactor design for  comparison.  As a result, the  user can optimize  toward a specific  design characteristic and review the results.

This program will  prove beneficial both as an  educational device for college level  science and engineering  students and as  a prelimininary conceptual design tool for scientists and engineers.

TABLE OF CONTENTS                                            Page

1.0 Introduction                                                                3

1.1 Background

1.2 Objective

1.3 Organization of Program

2.0 Using the Documentation                                            7

2.1 User's Manual

2.2 Help

2.3 Tutorials  Available From the Program

2.4 Data  in the Appendices

2.5 Saving Selections

3.0 Computer Hardware Compatibility                             9

3.1 Requirements for Running  Program

3.2 Color and Graphics Requirements

3.3 Printer Requirements

3.4 Making a Backup Copy of the Disk

4.0 Starting and Stopping the Program                          10

4.1 Getting Starting

4.2 Running the Demonstration

4.3 How to Exit the Program

5.0 DESIGN PHASE                                                     12

5.1.1 Selecting the Reactor Materials

5.1.2 Selecting Reactor Fuel

5.1.3 Selecting Reactor Moderator

5.1.4 Selecting Recator Coolant

5.2 Selecting the Reactor Geometry

5.2.1 Rectangular

5.2.2 Cylindrical

5.2.3 Spherical

5.3 Variables ( Input Parameters for System)

5.4 Design

5.5 Tools

5.5.1 Database

5.5.2 Maps

6.0 OUTPUT PHASE                                                    17

6.1 OUTPUT PHASE Menu

6.2 System Diagram

6.3 Reactor  Results

6.4 Primary Coolant Results

6.5 Secondary Coolant Results

6.6 System State Points

7.0 Failure to Achieve Criticality                                   19

8.0 Advanced Level DESIGN PHASE                           20

8.1 Input Your Own  Fuel Data

8.2 Input Your Own Moderator Data

Appendix A:  Data Tables                                             21

Appendix B: Assumptions and  Limitations                  23

Appendix C: Nuclear Reactor Examples                       24

1.0 INTRODUCTION

1.1 Background

The  development of  nuclear reactors  had  its origin  in the  atomic program of World  War II, known as the Manhattan  Project.  The reactors were designed to produce plutonium which was extracted and then used for atomic weapons.  The development of  nuclear reactors for commercial use originated with the  partners of the Manhattan Project; the  U.S.  , the U.K., France, and Canada.

Canada was the first nation to  emphasize the civil aspects of nuclear reactors.   It  had been  the  site  of major  experimental  facilities, including the  first reactor  outside of the  United States.   After the war, the U.K.  recalled her scientists from Canada to  begin the British nuclear program, but left Canada  with heavy water production facilities and vast resources of natural Uranium.  Canada's early research reactor, the NRX, developed  in 1947, was the precursor to  the present day CANDU heavy-water reactor.

The French  also set  up a  nuclear research  program and  their first reactor went  critical in 1948.  By  1956, they had a  plutonium powered reactor that was air-cooled and graphite moderated.

The U.K. began  their program with a gas-cooled,  MAGNOX type reactor. They  did not  want  to use  light water  reactors  because they  lacked Uranium enrichment facilities.

Unlike its three partners, the U.S.  had at that time large quantities of  alternative energy  resources  in  its coal,  oil,  and natural  gas deposits and so  did not immediately move ahead  with commercial reactor development.  As a  result, many different reactor  types were explored. Eventually,  the demand  for a  compact  and reliable  reactor for  U.S. submarines led to the rapid development of the Pressurized Water Reactor (PWR).  The PWR  and the Boiling Water Reactor  (BWR) eventually, became the basis of the U.S. commercial reactor industry.

As a result  of this historic development, there are  today, six basic types of commercial  nuclear reactors; (1) PWR, (2) BWR,  (3) CANDU, (4) Gas-Cooled  (MAGNOX),  (5)  Advanced   Gas-Cooled  (AGR)  and  (6)  High Temperature Gas Reactor (HTGR).

Besides the commercial reactors and the plutonium production reactors, there are numerous different types of experimental and research reactors as well as space reactors.  Their designs include fast reactors, aqueous fuel reactors, molten-salt reactors and breeder reactors.

The  design of  a THERMAL  NUCLEAR REACTOR  and aspects  of its  power conversion system is  a complex integration of  physics and engineering. It requires knowledge of neutron physics, thermal-hydraulics, materials, structures,  and more.   The choice  of a  reactor concept  is based  on extensive parametric studies that vary the selection of such fundamental variables as  fuel, moderator,  geometrical configuration,  coolant, and power  conversion system.   The objective  being to  optimize particular characteristics of the reactor in order  to meet the specifications of a special reactor design goal.

For example, the  objective of the efficient  production of electrical power for commerical power stations,  has led to nuclear reactor designs quite different than those reactors that  are to be deployment in space. "DESIGN YOUR  OWN NUCLEAR REACTOR"  has been composed  to advantageously combine  the  multiple  factors  involved  in  designing  very  new  and different  reactors,  into  a self-consistant  design,  which  minimizes conflicts.

1.2 Objective

The objective  of "DESIGN YOUR  OWN NUCLEAR  REACTOR" is to  perform a preliminary  conceptual  design  and  evaluation of  a  THERMAL  NUCLEAR REACTOR.  This  program should prove  beneficial both as  an educational device  for college  level science  and  engineering students  and as  a prelimininary conceptual design tool for scientists and engineers.

1.3 Organization of Program

"DESIGN  YOUR OWN  NUCLEAR  REACTOR"  is a  menu  driven program  that basically consists of two phases.  First, an DESIGN PHASE where the user can select  from various input possibilities  or input his own  data for various parameters for fuel, moderator, or coolant.  This is followed by the OUTPUT PHASE where the user  can review system diagrams with pop-out windows or lists  of calculated results of a  particular reactor design. In addition,  iterating through this  two phase process many  times will permit the user to accumulate a  variety of cases that can be contrasted and compared.

2.0 USING THE DOCUMENTATION

2.1 User's Manual

Sections 1.0 thru  8.0 of this User's Manual  provide instructions for the operation of the program "DESIGN YOUR OWN NUCLEAR REACTOR".  The Appendices provide useful for thermal nuclear reactor design.

2.2 Help

You may select Help from pull-down menu screens.  The Help screen will discuss the phases, data input, selection, and exit requirements.

FILES   DESIGN REACTOR     OUTPUT    HELP

Contents

About

2.3  Tutorials Available

From the  submenu under  Help from  the main menu.

From the DESIGN  PHASE screens, the user can call  up a brief tutorial on  certain aspects  of thermal  nuclear reactor  design.  Within  these tutorials, the basic  points involved in reactor  design are emphasized. Tutorials  are  available for  reactor  history,  materials and  design, equations and  assumptions,  and reactor examples.

2.4 Data in the Appendices

The  data  available   in  the  appendices  provides   the  user  with information about materials  used in reactor design  and, in particular, the data used within this program.

2.5 Saving Selections

In the main menu select Exit you will have an oportunity to save your selections to a file you designate. Later you can retrieve this design.

3.0 COMPUTER HARDWARE COMPATIBILITY

3.1 Computer Requirements for Running the Program

This program will run on an IBM PC or compatable under Windows3.1.

3.2 Color and Graphics Requirements

This program requires  a color monitor and adaptor  for color display, otherwise it will display black and white.

3.3 Printer Requirements

This program will print the output shown on the monitor to any printer supported by Windows 3.1.

3.4 Making a Backup Copy of the Disk

It is  recommended that you do  not use your program  disk for running problems, but rather make a back-up copy and use that disk instead.

4.0 STARTING AND STOPPING THE PROGRAM

4.1 Getting Started

After you have made a back-up copy  of the program disk, you can start "DESIGN YOUR OWN NUCLEAR REACTOR" by placing the back-up disk in drive A and from WINDOWS use

RUN "A:\REACTOR".

Click on the reactor icon to start thr program.  The Main menu  and main window will apprear.  The  pull-down menu offers  selection choices. In addition, command buttons as well as Click & Drag options exist.  These choices or menu selctions bring up another menu options. The Variables button or menu selection brings up a data  entry screen.  Help provides information on making selections and entering data.   The Reactor Results uses the DESIGN phase inputs and calculates and presents the results. Files/Quit ends the program. If you  immediately select Reactor Results  without entering data  or selecting fuel,  moderator, geometry  or coolant,  the program  will run  with the default reactor characteristics set.

4.2 Running the Demonstration Example

After starting the program  there  are default values  for  an

example reactor  design  to  run as  a demonstration.   The demonstration  program will  use the  default values  for a  reactor 1.0 meters cubed, with U-235 fuel, and  water as moderator and coolant.  The power conversion system will be closed loop.

When you select Reactor Results from the Menu or click the Reactor, Primary, or Secondary Buttons from the toolbar,the computer  will then quickly  calculate the critical fuel  mass and other characteristic variables . The  next three  choices are  summary output  pages of results for:  the Reactor, the  Primary Coolant, the  Secondary Coolant.  The selections will be listed in the menu. Review the results by selecting the the output options. You can exit a results page by selecting the EXIT button. You can print the page subsequently, by going to the Files / Print Page menu selction.

4.3 How to Exit the Program

You can exit the program from pull-down menu screen by selecting Files / Quit.

5.0 DESIGNING A REACTOR

During  the DESIGN REACTOR phase,  you  will have the following choices:

FILES  DESIGN REACTOR  REACTOR RESULTS   HELP

...

Materials

Geometry

Variables

Design Summary

Tools

5.1 DESIGN REACTOR Materials

If  you select Materials from the menu you will open submenus for fuel (as show below), moderator, or coolant options. Alternatively you could click & drag material choices from the materials toolbar and drop them over the appropriate reactor areas.

FILES  DESIGN REACTOR  REACTOR RESULTS   HELP

...

Materials.........Fuel................U-233

Geometry         Moderator    U-235

Variables          Coolant          Pu-239

Design Summary                    Pu-241

Tools                                       Cm-245

Am-242

5.1.1 Selecting Reactor Fuel

During the DESIGN  PHASE   select "Fuel" the Fuel selection menu  will appear.  You  can select  any of the  six available isotopes    presented    (Uranium-235,    Uranium-233,    Plutonium-239, Plutonium-241,  Americium-242, Curium-245)  or you  may define  your own fuel.  You  may select any of  the six isotopes offered  by pressing the associated number.   Notice that the  list of Current Selections  at the bottom of the page will reflect your  choice in fuel.  If you would like to input data for your own fuel and see section 8.1 for instructions.

NOTE: Atoms  with nuclei  containing the same  number of  protons, i.e., with  the same  atomic  number,  but with  different  mass numbers,  are essentially  identical   chemically,  although  they  can   have  widely different  nuclear properties.   Such  species, having  the same  atomic  number  but  different mass  numbers,  are  called isotopes.   The  fuel options are presented with the element name first (for example, Uranium)followed by the isotope number ( for example, 235 ).

5.1.2 Selecting Reactor Moderator

During the  DESIGN PHASE, if  you  select  Moderator, the Moderator selection  menu will appear.  You  can select any of  the five available materials presented (Water, Heavy Water, Beryllium, Carbon-12, and  Carbon-13) or  you may  choose  the sixth  option;"Define Your  Own Moderator".   You  may select  any  of  the  five materials  offered  by pressing the associated  letter or select with the arrow  and then press   RETURN.  Notice that the list of Current Selections at the bottom of the page will reflect your choice in  moderator.  If you would like to input data for  your own  moderator then  press "MD" and  see section  8.2 for instructions.

5.1.3 Selecting Reactor Coolant

During  the DESIGN  PHASE, if  you press  "C" or  select Coolant,  the  Coolant menu  selection will  appear.  You can  select a  combination of primary  and secondary  coolants:  water &  water,  hydrogen open  loop, helium  &   water,  and  sodium   &  water.   Selecting   hydrogen  will automatically result in an OPEN  LOOP power conversion system otherwise, a CLOSED LOOP power conversion system will  be used.

5.2 Selecting Reactor Geometry

During the  REACTOR DESIGN Phase,  if you select  Geometry from the menu or click on the Geometry button of the toolbar an input window will appear for dimensional input.

FILES  DESIGN REACTOR  REACTOR RESULTS   HELP

....

Moderator

Geometry................Rectangular

Variables                Cylindrical

Design Summary    Spherical

Tools

You can select  any of the three available  core configurations pesented (Rectangular Parallelpiped,  Cylindrical, or Spherical).  When  you have made a selection, a data input screen will appear.  You can input values for the appropriate dimensions (in  centimeters).  The input value range is provided  along with  the default  value under  the input  line.

5.2.1 Rectangular Geometry

For example,  if you choose reactangular  you input the length  in the x-direction, then the length in  the y-direction, and finally the length in the z-direction.

5.2.2 Cylindrical Geometry

For cylindrical, you will be asked for the cylindrical radius and height of the reactor.

5.2.3. Spherical Geometry

For spherical you will be asked for the radius.  The reactor geometry  assumes that the reactor is  a thermal homogeneous reactor and does NOT have a reflector region.  When you entered all data satisfactorily press RETURN to return to the DESIGN  PHASE menu.

5.3 Variables (Input Values for Power Conversion System)

During the DESIGN PHASE, if select Variables , the following input data screen will appear:

The program  starts with a set  of default values for  all parameters.

These values are listed in the  Appendices.  The program will keep these values until  you enter  a number  in an input  data screen.   Then your value will be used as long as the  program runs or until you enter a new value.

The range of allowable values appears  below the input line along with  the default value with which the program begins.

5.4 DESIGN

In the DESIGN Phase if you  select Design from the submenu a diplay screen of your current selections or default values is shown as illustrated below for the standard defaults:

5.5 TOOLS

There are several  tools available from within the  program.

FILES   DESIGN REACTOR    OUTPUT     HELP

Materials

Geometry

Variables

Design

5.5.1 Database (available only in special version.)

The database can be used to store earlier designs or reactor concepts or exiting reactor information. You can review, add, modify, list, sort, and print the items and data from the database.

5.5.2 Maps

Maps of  states  along with  their reactor locations is available.

6.0 OUTPUT PHASE

6.1 OUTPUT PHASE Menu

When you are satisfied with your design the DESIGN phase is over and you will next enter the OUTPUT PHASE by selecting RESULTS. The following is the OUTPUT PHASE menu:

FILES  DESIGN REACTOR  REPORT RESULTS  HELP

6.1.1 Diagram Results

If you select Diagram, a diagram  with results  for various components  of the  system.  Options 2  thru 5 present summary lists of results  for; the reactor, the Primary Coolant, the  Secondary  Coolant,  and  the  system  temperature,  pressure,  and flowrate state points, respectively.

The critical mass  of fuel in kilograms of the  pure isotope is given for a thermal, homogeneous, non-reflected  nuclear reactor.  You can use the advanced option  (see Section 8.1) for providing your  own fuel data to account for alloyed fuels of various enrichments.

The  annual   fuel  burnup  is   based  on  running  at   100%  power continuously, while assuming  a 0% breeding rate.  the reactor geometric buckling  is computed  using the  formular in  section 9.0.  the reactor moderator to fuel ratio is  calculated using the Fermi age, homogeneous,criticality one-group, thermal equation.

6.1.2 Reactor Results

During  the OUTPUT  PHASE, if  you select Reactor,  the

following list of variables will be presented: (the values presented are the results of  calculations for the default reactor  parameters used in the example problem.)

6.1.3 Primary Coolant Results

During  the OUTPUT  PHASE, if  you  select Primary,  the

following list of variables will be presented: (the values presented are the results of  calculations for the default reactor  parameters used in the example problem.)

6.1.4 Secondary Coolant Results

During the OUTPUT PHASE, if you select REACTOR RESULTS / Secondary, from the menu or click on the Results Secondary button a list  of  the secondary coolant variables  will  be  presented  (the  initial values presented  are  the results  of  calculations  for the  default  reactor parameters used in the example problem.)

7.0 FAILURE TO ACHIEVE CRITICALITY

If  your  selections  of  materials  and  geometry  can  not  achieve criticality a warning screen will appear and you will be returned to the DESIGN PHASE in order for you to change the selections until you reach a critical configuration.

8.0 ADVANCED LEVEL

8.1 Input Your Own Fuel Data

During the  DESIGN PHASE, if you select Fuel  /  Define Your Own Fuel, or click & drag the Design Fuel icon over to the fuel selection area the list  of fuel variables  will be presented  for your  input.

Values of these parameters for  the fuels available in this program, are presented in Table I of Appendix A.

8.2 Input Your Own Moderator Data

During the DESIGN PHASE, if you select Moderator / Define Your Own Moderator or  click & drag the Design Mod icon over to the Moderator selection area  a list of variables will  be presented  for your input.

Values of these parameters for  the moderators available in this program, are presented in Table II of Appendix A.

Values  for these  parameters for  the moderators  available in  this program, are presented in Table II of Appendix A.

APPENDIX A

DATA TABLES

I. Reactor Fuels

isotope	density	Mass	Eta	Xfiss	Xabs

_______	_______	____	____	_____	_____

g/cc			barns	barns

U-233	18.9	233	2.27	524	45

U-235	18.9	235	2.06	577	106

Pu-239	19.0	239	2.10	760	287

Pu-241	19.0	241	2.06	988	378

Cm-245	12.0	245	2.00	6000	1677

Am-242	14.0	242	2.00	1700	331

II. Reactor Moderators

Mat	Diffusion	Fermi	Density	Xabs

_______	_________	_______	_______	____

Water	    7.6	   31	   1	.66

HeavyH2O	10000	  121	   1.1	.002

Berylium	  441	   84	   1.85	.01

Carbon12	 4121	  350	   1.9	.004

Carbon13	 4121	  350	   1.9	.002

III. Reactor Coolants

Water	Hydrogen	Helium

Heat Cap	1 to 1.2	1 to 3.95	1 to 1.1

Density 	1	0.009	0.009

IV. Default Values

Fuel	Uranium 235

Moderator	Water

Primary Coolant	Water

Secondary Coolant	Water

Geometry	1m x 1m x 1m

Coolant Pressure 	92 atmospheres

Porosity	.33

Coolant Flowrate	10000 kg/sec

Coolant Loop	Closed

Coolant Tube dia.	0.4 cm

APPENDIX :B

PROGRAM ASSUMPTIONS AND LIMITATIONS

1.  Thermal  group approximation, homogeneous reator  approximation, and Fermi age approximation were used to compute criticality.

2.  Maximum gas velocity of MACH 0.3 was used.

APPENDIC :C

NUCLEAR REACTOR EXAMPLES

EXAMPLES	    PWR	     HWR	    Units

__________	__________	__________

Oconee 1	Pickering

Power (th)	    2584	    1742	    MW(t)

Power 	     886	     508	    MW(e)

Volume	      30.7	     265	    m^3

Fuel	     UO2	     UO2

Mass	   83000	   92600	    kg

Cool.Temp.	 563-591	 523-567	    deg. K

Flowrate	   12600	   17700	    kg/sec

Power Den.	      31.1   	      10	    w/g

Heat Flux	      54.1	      59.2	    w/cm^2

Sub Form_Load ()

NL = Chr$(10) + Chr$(13)

For i = 1 To 13

save(i) = "            "

Next i

Powerth = 100

crossfiss = 582

crossabs1 = 106

thermalconduct = 14

crosscatf = 15

fcrossabs = 679

densityf = 19

Af = 235

diffus = 7.617

fermi = 31.36

densitym = 1

mcrossabs = .66

ccrossabs = .66

Am = 18

LX = 100

LY = 100

LZ = 100

cr = 50

r = 50

cheight = 100

Volume = (LX * LY * LZ) / (1000000)

Buckling = ((pi / LX) ^ 2 + (pi / LY) ^ 2 + (pi / LZ) ^ 2)

porosity = .33

tubedia = .4

flowrate = 10000

tubelength = 100

Effic = 34

numbertubes = 2600

maxtemp = 530

mintemp = 470

FuelTemp = 877

namefuel = "U-235"

namemod = "H2O"

namecool1 = "H2O"

namecool2 = "H2O"

namegeo = "Rectangular "

pressurec = 46 * safetyfactor

viscosity = .16

Pri_Pressure = 100

Sec_Temperature(4) = 516

Sec_Pressure(4) = 34.9

Sec_Enthalpy(4) = 1205

Sec_Temperature(5) = 52

Sec_Pressure(5) = .1436

Sec_Enthalpy(5) = 850

Sec_Temperature(6) = 41

Sec_Pressure(6) = .0769

Sec_Enthalpy(6) = 68

Sec_Temperature(7) = 104

Sec_Pressure(7) = 34.9

Sec_Enthalpy(7) = 168

tubearea = (pi * tubedia * tubelength * numbertubes)

heatflux = (Powerth * 1000000) / (tubearea)

image2.Picture = GroupPush3D19.PictureUp

image3.Picture = GroupPush3D13.PictureUp

image4.Picture = GroupPush3D8.PictureUp

image5.Picture = GroupPush3D13.PictureUp

image6.Picture = GroupPush3D13.PictureUp

image12.Picture = GroupPush3D13.PictureUp

image11.Picture = GroupPush3D19.PictureUp

image8.Picture = GroupPush3D13.PictureUp

image7.Picture = GroupPush3D8.PictureUp

image9.Picture = GroupPush3D13.PictureUp

image10.Picture = GroupPush3D13.PictureUp

Calculation_Phase

End Sub

Sub GroupPush3D1_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image10.Picture = GroupPush3D1.PictureUp

image9.Picture = GroupPush3D1.PictureUp

GroupPush3D1.Drag 1

dragx = X

dragy = Y

Buffernamecool2 = "H2"

Buffernamecool1 = "H2"

End Sub

Sub GroupPush3D1_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D1.Drag 2

End Sub

Sub GroupPush3D10_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image7.Picture = GroupPush3D10.PictureUp

GroupPush3D10.Drag 1

dragx = X

dragy = Y

Buffernamefuel = "Pu-241"

End Sub

Sub GroupPush3D10_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D10.Drag 2

End Sub

Sub GroupPush3D11_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image7.Picture = GroupPush3D11.PictureUp

GroupPush3D11.Drag 1

dragx = X

dragy = Y

Buffernamefuel = "Cm-245"

End Sub

Sub GroupPush3D11_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D11.Drag 2

End Sub

Sub GroupPush3D12_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image7.Picture = GroupPush3D12.PictureUp

GroupPush3D12.Drag 1

dragx = X

dragy = Y

Buffernamefuel = "Am-242"

End Sub

Sub GroupPush3D12_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D12.Drag 2

End Sub

Sub GroupPush3D13_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image10.Picture = GroupPush3D13.PictureUp

image9.Picture = GroupPush3D13.PictureUp

image8.Picture = GroupPush3D13.PictureUp

GroupPush3D13.Drag 1

dragx = X

dragy = Y

Buffernamecool1 = "H2O"

Buffernamecool2 = "H2O"

Buffernamemod = "H2O"

End Sub

Sub GroupPush3D13_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D13.Drag 2

End Sub

Sub GroupPush3D14_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image8.Picture = GroupPush3D14.PictureUp

GroupPush3D14.Drag 1

dragx = X

dragy = Y

Buffernamemod = "D2O"

End Sub

Sub GroupPush3D14_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D14.Drag 2

End Sub

Sub GroupPush3D15_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image8.Picture = GroupPush3D15.PictureUp

GroupPush3D15.Drag 1

dragx = X

dragy = Y

Buffernamemod = "Designmod"

End Sub

Sub GroupPush3D15_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D15.Drag 2

End Sub

Sub GroupPush3D16_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image7.Picture = GroupPush3D16.PictureUp

GroupPush3D16.Drag 1

dragx = X

dragy = Y

Buffernamefuel = "Designfuel"

End Sub

Sub GroupPush3D16_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D16.Drag 2

End Sub

Sub GroupPush3D17_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image11.Picture = GroupPush3D17.PictureUp

GroupPush3D17.Drag 1

dragx = X

dragy = Y

Buffernamegeo = "Spherical   "

MnuSpherical_click

image1.Visible = True

image17.Visible = True

image18.Visible = False

image19.Visible = False

End Sub

Sub GroupPush3D17_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D17.Drag 2

End Sub

Sub GroupPush3D18_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image11.Picture = GroupPush3D18.PictureUp

GroupPush3D18.Drag 1

dragx = X

dragy = Y

Buffernamegeo = "Cylindrical "

MnuCylindrical_click

image1.Visible = True

image17.Visible = False

image18.Visible = True

image19.Visible = False

End Sub

Sub GroupPush3D18_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D18.Drag 2

End Sub

Sub GroupPush3D19_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image11.Picture = GroupPush3D19.PictureUp

GroupPush3D19.Drag 1

dragx = X

dragy = Y

Buffernamegeo = "Rectangular "

MnuRectangular_click

image1.Visible = True

image17.Visible = False

image18.Visible = False

image19.Visible = True

End Sub

Sub GroupPush3D19_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D19.Drag 2

End Sub

Sub GroupPush3D2_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image9.Picture = GroupPush3D2.PictureUp

GroupPush3D2.Drag 1

dragx = X

dragy = Y

Buffernamecool1 = "Helium"

End Sub

Sub GroupPush3D2_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D2.Drag 2

End Sub

Sub GroupPush3D20_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image13.Visible = False

image16.Visible = False

MnuHydrogen_click

End Sub

Sub GroupPush3D21_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image13.Visible = True

image16.Visible = True

image5.Picture = GroupPush3D13.PictureUp

image6.Picture = GroupPush3D13.PictureUp

namecool1 = "H2O"

namecool2 = "H2O"

MnuWater_click

End Sub

Sub GroupPush3D22_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

INPUTTEXT1 = "INPUT Variables"

INPUTTEXT2 = "Maximum Coolant Temperature "

inputtext3 = maxtemp & "  degree K"

InputText4 = "Minimum Coolant Temperature"

inputtext5 = mintemp & "  degree K"

InputText6 = "Coolant Tube Diameter"

inputtext7 = tubedia & "  cm"

InputText8 = "Reactor Porosity"

inputtext9 = porosity

InputText10 = "Coolant Flowrate"

inputtext11 = flowrate & "  kg/s"

FrmInput.Show 1

maxtemp = Val(inputtext3)

mintemp = Val(inputtext5)

tubedia = Val(inputtext7)

porosity = Val(inputtext9)

flowrate = Val(inputtext11)

End Sub

Sub GroupPush3D23_Click (Value As Integer)

calculator.Show

End Sub

Sub GroupPush3D24_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

FrmMap.Show

End Sub

Sub GroupPush3D25_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

MnuReactorResults_Click

End Sub

Sub GroupPush3D26_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

MnuPrimaryCoolant_Click

End Sub

Sub GroupPush3D27_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

MnuSecCoolant_Click

End Sub

Sub GroupPush3D3_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image8.Picture = GroupPush3D3.PictureUp

GroupPush3D3.Drag 1

dragx = X

dragy = Y

Buffernamemod = "Berylium"

End Sub

Sub GroupPush3D3_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D3.Drag 2

End Sub

Sub GroupPush3D4_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image9.Picture = GroupPush3D4.PictureUp

GroupPush3D4.Drag 1

dragx = X

dragy = Y

Buffernamecool1 = "Sodium"

End Sub

Sub GroupPush3D4_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D4.Drag 2

End Sub

Sub GroupPush3D5_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image8.Picture = GroupPush3D5.PictureUp

GroupPush3D5.Drag 1

dragx = X

dragy = Y

Buffernamemod = "Carbon-12"

End Sub

Sub GroupPush3D5_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D5.Drag 2

End Sub

Sub GroupPush3D6_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image8.Picture = GroupPush3D6.PictureUp

GroupPush3D6.Drag 1

dragx = X

dragy = Y

Buffernamemod = "Carbon-13"

End Sub

Sub GroupPush3D6_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D6.Drag 2

End Sub

Sub GroupPush3D7_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image7.Picture = GroupPush3D7.PictureUp

GroupPush3D7.Drag 1

dragx = X

dragy = Y

Buffernamefuel = "U-233"

End Sub

Sub GroupPush3D7_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D7.Drag 2

End Sub

Sub GroupPush3D8_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image7.Picture = GroupPush3D8.PictureUp

GroupPush3D8.Drag 1

dragx = X

dragy = Y

Buffernamefuel = "U-235"

End Sub

Sub GroupPush3D8_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D8.Drag 2

End Sub

Sub GroupPush3D9_MouseDown (Button As Integer, Shift As Integer, X As Single, Y As Single)

image7.Picture = GroupPush3D9.PictureUp

GroupPush3D9.Drag 1

dragx = X

dragy = Y

Buffernamefuel = "Pu-239"

End Sub

Sub GroupPush3D9_MouseUp (Button As Integer, Shift As Integer, X As Single, Y As Single)

GroupPush3D9.Drag 2

End Sub

Sub Image12_DragDrop (Source As Control, X As Single, Y As Single)

image12.Picture = image8.Picture

image3.Picture = image8.Picture

If Buffernamemod = "Berylium" Then

MnuBerylium_Click

End If

If Buffernamemod = "Carbon-12" Then

MnuCarbon12_click

End If

If Buffernamemod = "Carbon-13" Then

MnuCarbon13_CLICK

End If

If Buffernamemod = "H2O" Then

MnuWater_click

End If

If Buffernamemod = "D2O" Then

MnuHeavyWater_CLICK

End If

If Buffernamemod = "Designmod" Then

MnuDesignYourOwnMod_Click

Buffernamemod = "            "

End If

End Sub

Sub Image2_DragDrop (Source As Control, X As Single, Y As Single)

image2.Picture = image11.Picture

If Buffernamegeo = "Spherical" Then

MnuSpherical_click

End If

If Buffernamegeo = "Cylindrical" Then

MnuCylindrical_click

End If

If Buffernamegeo = "Rectangular" Then

MnuRectangular_click

End If

End Sub

Sub Image3_DragDrop (Source As Control, X As Single, Y As Single)

image12.Picture = image8.Picture

image3.Picture = image8.Picture

If Buffernamemod = "Berylium" Then

MnuBerylium_Click

End If

If Buffernamemod = "Carbon-12" Then

MnuCarbon12_click

End If

If Buffernamemod = "Carbon-13" Then

MnuCarbon13_CLICK

End If

If Buffernamemod = "H2O" Then

MnuWater_click

End If

If Buffernamemod = "D2O" Then

MnuHeavyWater_CLICK

End If

If Buffernamemod = "Designmod" Then

MnuDesignYourOwnMod_Click

Buffernamemod = "            "

End If

End Sub

Sub Image4_DragDrop (Source As Control, X As Single, Y As Single)

image4.Picture = image7.Picture

If Buffernamefuel = "U-233" Then

MnuU233_click

End If

If Buffernamefuel = "U-235" Then

MnuU235_click

End If

If Buffernamefuel = "Pu-239" Then

MnuPlutonium239_click

End If

If Buffernamefuel = "Pu-241" Then

MnuPlutonium241_click

End If

If Buffernamefuel = "Cm-245" Then

MnuCurium245_click

End If

If Buffernamefuel = "Am-242" Then

MnuAmericium242_click

End If

If Buffernamefuel = "Designfuel" Then

MnuDesignYourOwnFuel_Click

Buffernamefuel = "            "

End If

End Sub

Sub Image5_DragDrop (Source As Control, X As Single, Y As Single)

image5.Picture = image9.Picture

If Buffernamecool1 = "Hydrogen" Then

MnuHydrogen_click

End If

If Buffernamecool1 = "Helium" Then

MnuHelium_click

End If

If Buffernamecool1 = "Sodium" Then

MnuSodium_click

End If

If Buffernamecool1 = "H2O" Then

MnuWater_click

End If

End Sub

Sub Image6_DragDrop (Source As Control, X As Single, Y As Single)

image6.Picture = image10.Picture

If Buffernamecool2 = "H2O" Then

MnuWater_click

End If

End Sub

Sub MnuAbout_Click ()

Aboutbox.Show 1

End Sub

Sub MnuAmericium242_click ()

namefuel = "Am-242"

crossfiss = 6000

crosscatf = 12

fcrossabs = 7700

densityf = 19

Af = 242

thermalconduct = 14

image4.Picture = GroupPush3D12.PictureUp

End Sub

Sub MnuBerylium_Click ()

namemod = "Be"

diffus = 441

fermi = 84.64

densitym = 1.85

mcrossabs = .01

Am = 9

image3.Picture = GroupPush3D3.PictureUp

image12.Picture = GroupPush3D3.PictureUp

End Sub

Sub MnuCalculator_Click ()

calculator.Show

End Sub

Sub MnuCarbon12_click ()

namemod = "C-12"

diffus = 4121

fermi = 350

densitym = 1.9

mcrossabs = .004

Am = 12

image3.Picture = GroupPush3D5.PictureUp

image12.Picture = GroupPush3D5.PictureUp

End Sub

Sub MnuCarbon13_CLICK ()

namemod = "C-13"

diffus = 4121

fermi = 350

densitym = 1.9

mcrossabs = .002

Am = 13

image3.Picture = GroupPush3D6.PictureUp

image12.Picture = GroupPush3D6.PictureUp

End Sub

Sub MnuContents_Click ()

winhelp hWnd, help_file, help_context, HELP_INDEX

'app.HelpFile = "c:\vb\apply\rx\bj.hlp"

End Sub

Sub MnuCoolantWater_Click ()

water

image5.Picture = GroupPush3D13.PictureUp

image6.Picture = GroupPush3D13.PictureUp

image13.Visible = True

image16.Visible = True

End Sub

Sub MnuCurium245_click ()

namefuel = "Cm-245"

crossfiss = 1700

crosscatf = 20

fcrossabs = 2000

densityf = 19

Af = 245

thermalconduct = 14

image4.Picture = GroupPush3D11.PictureUp

End Sub

Sub MnuCylindrical_click ()

INPUTTEXT1 = "INPUT"

INPUTTEXT2 = "Set Reacor Radius"

inputtext3 = cr & "  cm"

InputText4 = "Set Reactor Height"

inputtext5 = cheight & "  cm"

InputText6 = ""

inputtext7 = ""

InputText8 = ""

inputtext9 = ""

InputText10 = ""

inputtext11 = ""

InputText12 = ""

inputtext13 = ""

FrmInput.Show 1

cr = Val(inputtext3)

cheight = Val(inputtext5)

Volume = Val(inputtext5) * pi * ((Val(inputtext3)) ^ 2) / 1000000

namegeo = "Cylindrical "

Buckling = (2.405 / (Val(inputtext3))) ^ 2 + (pi / (Val(inputtext5))) ^ 2

image2.Picture = GroupPush3D18.PictureUp

image1.Visible = True

image17.Visible = False

image18.Visible = True

image19.Visible = False

End Sub

Sub MnuDesignSummary_Click ()

outputtext1 = "DESIGN SUMMARY"

If namegeo = "Rectangular " Then

OUTPUTTEXT2 = "Length "

OUTPUTTEXT3 = LX & "  cm"

OUTPUTTEXT4 = "Height"

OUTPUTTEXT5 = LY & "  cm"

OUTPUTTEXT6 = "Width"

OUTPUTTEXT7 = LZ & "  cm"

End If

' Format(tubedia, "0.00E+00") & "  cm"

If namegeo = "Cylindrical " Then

OUTPUTTEXT2 = "Cylindrical Radius"

OUTPUTTEXT3 = cr & "  cm"

OUTPUTTEXT4 = "Height"

OUTPUTTEXT5 = cheight & "  cm"

OUTPUTTEXT6 = ""

OUTPUTTEXT7 = ""

End If

If namegeo = "Spherical   " Then

OUTPUTTEXT2 = "Spherical Radius"

OUTPUTTEXT3 = r & "  cm"

OUTPUTTEXT4 = ""

OUTPUTTEXT5 = ""

OUTPUTTEXT6 = ""

OUTPUTTEXT7 = ""

End If

OUTPUTTEXT8 = "Fuel"

OUTPUTTEXT9 = namefuel

OUTPUTTEXT10 = "Moderator"

OUTPUTTEXT11 = namemod

OUTPUTTEXT12 = "Primary Coolant"

OUTPUTTEXT13 = namecool1

OUTPUTTEXT14 = "Secondary Coolant"

OUTPUTTEXT15 = namecool2

OUTPUTTEXT16 = "Maximum Temperature "

OUTPUTTEXT17 = maxtemp & "  degree K"

OUTPUTTEXT18 = "Minimum Temperature "

OUTPUTTEXT19 = mintemp & "  degree K"

OUTPUTTEXT20 = "Coolant Tube Diameter "

OUTPUTTEXT21 = tubedia & "  cm"

OUTPUTTEXT22 = "Porosity "

OUTPUTTEXT23 = porosity

OUTPUTTEXT24 = "Coolant Flowrate"

OUTPUTTEXT25 = flowrate & "  kg/s"

frmout.Show 1

End Sub

Sub MnuDesignYourOwnFuel_Click ()

INPUTTEXT1 = "DESIGN YOUR OWN NUCLEAR FUEL INPUT"

INPUTTEXT2 = "Thermal Fission Cross Section"

inputtext3 = crossfiss & "  barns"

InputText4 = "Thermal Absorption Cross Section"

inputtext5 = fcrossabs & "  barns"

InputText6 = "Thermal Conductivity"

inputtext7 = thermalconduct & "  BTU/hr.ft.F"

InputText8 = "Density"

inputtext9 = densityf & " g/cc"

InputText10 = "Atomic Mass Number"

inputtext11 = Af

InputText12 = "Fuel Name"

inputtext13 = namefuel

FrmInput.Show 1

crossfiss = Val(inputtext3)

fcrossabs = Val(inputtext5)

thermalconduct = Val(inputtext7)

densityf = Val(inputtext9)

Af = Val(inputtext11)

namefuel = inputtext13

image4.Picture = GroupPush3D16.PictureUp

End Sub

Sub MnuDesignYourOwnMod_Click ()

INPUTTEXT1 = "DESIGN YOUR OWN MODERATOR"

INPUTTEXT2 = "Diffusion Length Squared"

inputtext3 = diffus & "  cm^-2"

InputText4 = "Fermi Age"

inputtext5 = fermi & "  cm^-2"

InputText6 = "Thermal Absorption Cross Section"

inputtext7 = mcrossabs & "  barns"

InputText8 = "Density"

inputtext9 = densitym & "  g/cc"

InputText10 = "Mass Number"

inputtext11 = Am

InputText12 = "Moderator Name"

inputtext13 = namemod

FrmInput.Show 1

diffus = Val(inputtext3)

fermi = Val(inputtext5)

mcrossabs = Val(inputtext7)

densitym = Val(inputtext9)

Am = Val(inputtext11)

namemod = inputtext13

image3.Picture = GroupPush3D15.PictureUp

image12.Picture = GroupPush3D15.PictureUp

End Sub

Sub MnuFileOpen_Click ()

On Error GoTo ErrHandler

CMDialog1.Filter = "All Files (*.*)|*.*|Text Files (*.txt)|*.txt|Batch Files (*.bat)|*.bat"

CMDialog1.FilterIndex = 2

CMDialog1.Action = 1

myfile = CMDialog1.Filename

Open myfile For Binary As #1

For i = 1 To 13

Get #1, , save(i)

Next i

Close #1

ErrHandler:

namefuel = save(1)

namemod = save(2)

namecool1 = save(3)

namecool2 = save(4)

namegeo = save(5)

maxtemp = Val(save(6))

mintemp = Val(save(7))

tubedia = Val(save(8))

porosity = Val(save(9))

flowrate = Val(save(10))

If namegeo = "Rectangular " Then

LX = Val(save(11))

LY = Val(save(12))

LZ = Val(save(13))

Volume = LX * LY * LZ / (1000000)

Buckling = ((pi / LX) ^ 2) + ((pi / LY) ^ 2) + ((pi / LZ) ^ 2)

End If

If namegeo = "Cylindrical " Then

cr = Val(save(11))

cheight = Val(save(12))

Volume = (pi * (cr * cr) * cheight) / (1000000)

tubelength = cr

Buckling = ((2.405 / cr) ^ 2 + (pi / cheight) ^ 2)

End If

If namegeo = "Spherical   " Then

r = Val(save(11))

Volume = (1.33 * pi * r * r * r) / (1000000)

tubelength = r

Buckling = (pi / r) ^ 2

End If

'End If

'If (lx > ly) And (lx > lz) Then

'tubelength = lx

'End If

' If (ly > lx) And (ly > lz) Then

'tubelength = ly

'  End If

'If (lz > lx) And (lz > ly) Then

' tubelength = lz

If namefuel = "U-233       " Then

MnuU233_click

End If

If namefuel = "U-235       " Then

MnuU235_click

End If

If namefuel = "Pu-239      " Then

MnuPlutonium239_click

End If

If namefuel = "Pu-241      " Then

MnuPlutonium241_click

End If

If namefuel = "Am-242      " Then

MnuAmericium242_click

End If

If namefuel = "Cm-24       " Then

MnuCurium245_click

End If

If namemod = "H2O         " Then

MnuWater_click

End If

If namemod = "D2O         " Then

MnuHeavyWater_CLICK

End If

If namemod = "Be          " Then

MnuBerylium_Click

End If

If namemod = "C-12        " Then

MnuCarbon12_click

End If

If namemod = "C-13        " Then

MnuCarbon13_CLICK

End If

If namecool1 = "H2O         " Then

MnuWater_click

End If

If namecool1 = "H2          " Then

MnuHydrogen_click

End If

If namecool1 = "He          " Then

MnuHelium_click

End If

If namecool1 = "SODIUM      " Then

MnuSodium_click

End If

Calculation_Phase

End Sub

Sub MnuFileSave_Click ()

save(1) = namefuel

save(2) = namemod

save(3) = namecool1

save(4) = namecool2

save(5) = namegeo

save(6) = Str(maxtemp)

save(7) = Str(mintemp)

save(8) = Str(tubedia)

save(9) = Str(porosity)

save(10) = Str(flowrate)

If namegeo = "Rectangular " Then

save(11) = Str(LX)

save(12) = Str(LY)

save(13) = Str(LZ)

End If

If namegeo = "Cylindrical " Then

save(11) = Str(cr)

save(12) = Str(cheight)

save(13) = "            "

End If

If namegeo = "Spherical   " Then

save(11) = Str(r)

save(12) = "            "

save(13) = "            "

End If

On Error GoTo ErrHandler1

CMDialog1.Filter = "All Files (*.*)|*.*|Text Files (*.txt)|*.txt|Batch Files (*.bat)|*.bat"

CMDialog1.FilterIndex = 2

CMDialog1.Action = 2

myfile = CMDialog1.Filename

Open myfile For Binary As #1

For i = 1 To 13

Put #1, , save(i)

Next i

Close #1

ErrHandler1:

Exit Sub

End Sub

Sub MnuHeavyWater_CLICK ()

namemod = "D2O"

diffus = 10000

fermi = 121

densitym = 1.1

mcrossabs = .002

Am = 20

image3.Picture = GroupPush3D14.PictureUp

image12.Picture = GroupPush3D14.PictureUp

End Sub

Sub MnuHelium_click ()

image13.Visible = True

image16.Visible = True

namecool1 = "Helium"

namecool2 = "H2O"

helium

image5.Picture = GroupPush3D2.PictureUp

image6.Picture = GroupPush3D13.PictureUp

End Sub

Sub MnuHydrogen_click ()

image13.Visible = False

image16.Visible = False

namecool1 = "Hydrogen"

namecool2 = "Hydrogen"

hydrogen

image5.Picture = GroupPush3D1.PictureUp

image6.Picture = GroupPush3D1.PictureUp

End Sub

Sub MnuMap_Click ()

FrmMap.Show 1

End Sub

Sub MnuPlutonium239_click ()

namefuel = "Pu-239"

crossfiss = 760

crosscatf = 11

fcrossabs = 1030

densityf = 19

Af = 239

thermalconduct = 14

image4.Picture = GroupPush3D9.PictureUp

End Sub

Sub MnuPlutonium241_click ()

namefuel = "Pu-241"

crossfiss = 988

crosscatf = 10

fcrossabs = 1384

densityf = 19

Af = 241

thermalconduct = 14

image4.Picture = GroupPush3D10.PictureUp

End Sub

Sub MnuPrimaryCoolant_Click ()

Calculation_Phase

outputtext1 = "SUMMARY OF PRIMARY COOLANT RESULTS"

OUTPUTTEXT2 = "Reactor Porosity or Void Fraction"

OUTPUTTEXT3 = Format(porosity, "0.00E+00")

OUTPUTTEXT4 = "Coolant Tube Diameter"

OUTPUTTEXT5 = Format(tubedia, "0.00E+00") & "  cm"

OUTPUTTEXT6 = "Number Of Coolant Channels"

OUTPUTTEXT7 = Format(numbertubes, "0.00E+00")

OUTPUTTEXT8 = "Primary Coolant Pressure"

OUTPUTTEXT9 = Format(Pri_Pressure, "0.00E+00") & "  atmos"

OUTPUTTEXT10 = "Primary Coolant Flowrate"

OUTPUTTEXT11 = Format(flowrate, "0.00E+00") & "  kg/s"

OUTPUTTEXT12 = "Primary Coolant Velocity"

OUTPUTTEXT13 = Format(velocity, "0.00E+00") & "  m/s"

OUTPUTTEXT14 = "Reactor Pressure Drop"

OUTPUTTEXT15 = Format(Rx_Pressure_Drop, "0.00E+00") & "  atmos"

OUTPUTTEXT16 = "Reactor Pump Power"

OUTPUTTEXT17 = Format(Pump_Power, "0.00E+00") & "  MWe"

OUTPUTTEXT18 = "Coolant Mass within Reactor"

OUTPUTTEXT19 = Format(massc, "0.00E+00") & "  Mg " & namecool1

OUTPUTTEXT20 = "Max Primary Coolant Temperature"

OUTPUTTEXT21 = Format(maxtemp, "0.00E+00") & "  degree K"

OUTPUTTEXT22 = "Differential Coolant Temperature"

OUTPUTTEXT23 = Format(DeltaT, "0.00E+00") & "  degree K"

OUTPUTTEXT24 = ""

OUTPUTTEXT25 = ""

frmout.Show 1

End Sub

Sub MnuPrint_Click ()

On Error GoTo ErrHandler2

CMDialog1.Filter = "All Files (*.*)|*.*|Text Files (*.txt)|*.txt|Batch Files (*.bat)|*.bat"

CMDialog1.FilterIndex = 2

CMDialog1.Action = 5

BeginPage = CMDialog1.FromPage

EndPage = CMDialog1.ToPage

NumCopies = CMDialog1.Copies

For i = 1 To NumCopies

' sent to pRINTER

frmout.PrintForm

Next

ErrHandler2:

Exit Sub

End Sub

Sub MnuQuit_Click ()

winhelp hWnd, help_file, Help_Quit, 0

Unload frmmain

End

End Sub

Sub MnuReactorResults_Click ()

Calculation_Phase

outputtext1 = "SUMMARY OF NUCLEAR REACTOR DESIGN RESULTS" & Chr(13) & Chr(10) & "hhhhhh"

OUTPUTTEXT2 = "Volume of Reactor"

OUTPUTTEXT3 = Format(Volume, "0.00E+00") & "  m^3 "

OUTPUTTEXT4 = "Reactor Geometric Buckling"

OUTPUTTEXT5 = Format(Buckling, "0.00E+00") & "  cm^-2"

OUTPUTTEXT6 = "Moderator-to-Fuel Ratio"

OUTPUTTEXT7 = Format(ratio, "0.00E+00") & "  "

OUTPUTTEXT8 = "Fuel Number Density"

OUTPUTTEXT9 = Format(nf, "0.00E+00") & "  atoms/cc"

OUTPUTTEXT10 = "Moderator Mass"

OUTPUTTEXT11 = Format(massm, "0.00E+00") & "  MG " & namemod

OUTPUTTEXT12 = "Critical Mass"

OUTPUTTEXT13 = Format(critmass, "0.00E+00") & "  kg " & namefuel

OUTPUTTEXT14 = "Annual Fuel Burnup"

OUTPUTTEXT15 = Format(burnup, "0.00E+00") & "  Mg/yr " & namefuel

OUTPUTTEXT16 = "Specific Power"

OUTPUTTEXT17 = Format(specificpower, "0.00E+00") & " kw/gram " & namefuel

OUTPUTTEXT18 = "Avg. Heat Flux"

OUTPUTTEXT19 = Format(heatflux, "0.00E+00") & "  watts/cm^2"

OUTPUTTEXT20 = "Maximum Fuel Temperature"

OUTPUTTEXT21 = Format(FuelTemp, "0.00E+00") & "  degree K"

OUTPUTTEXT22 = "Avg. Thermal Neutron Flux"

OUTPUTTEXT23 = Format(N_Flux, "0.00E+00") & " /sec-cm^2"

OUTPUTTEXT24 = "Thermal Power Produced"

OUTPUTTEXT25 = Format(Powerth, "0.00E+00") & "  MWth "

frmout.Show 1

End Sub

Sub MnuRectangular_click ()

INPUTTEXT1 = "INPUT"

INPUTTEXT2 = "Set Reacor Length"

inputtext3 = LX & "  cm"

InputText4 = "Set Reactor Width"

inputtext5 = LY & "  cm"

InputText6 = "Set Reactor Height"

inputtext7 = LZ & "  cm"

InputText8 = ""

inputtext9 = ""

InputText10 = ""

inputtext11 = ""

InputText12 = ""

inputtext13 = ""

FrmInput.Show 1

LX = Val(inputtext3)

LY = Val(inputtext5)

LZ = Val(inputtext7)

Volume = (Val(inputtext3) * Val(inputtext5) * Val(inputtext7) / 1000000)

namegeo = "Rectangular "

tubelength = Val(inputtext3)

Buckling = ((pi / Val(inputtext3)) ^ 2 + (pi / Val(inputtext5)) ^ 2 + (pi / Val(inputtext7)) ^ 2)

image2.Picture = GroupPush3D19.PictureUp

image1.Visible = True

image17.Visible = False

image18.Visible = True

image19.Visible = True

End Sub

Sub MnuSecCoolant_Click ()

Calculation_Phase

outputtext1 = "SUMMARY OF SECONDARY COOLANT RESULTS"

OUTPUTTEXT2 = "Steam Temerature "

OUTPUTTEXT3 = Format(Sec_Temperature(4), "0.00E+0") & "  degree K"

OUTPUTTEXT4 = "Steam generator Pressure(4) "

OUTPUTTEXT5 = Format(Sec_Pressure(4), "0.00E+0") & "  atmos"

OUTPUTTEXT6 = "Turbine Efficiency"

OUTPUTTEXT7 = Format(80, "0.00E+0") & "  %"

OUTPUTTEXT8 = "Secondary Efficiency "

OUTPUTTEXT9 = Format(Effic, "0.00E+0") & "  %"

OUTPUTTEXT10 = "Condensate Pump Power"

OUTPUTTEXT11 = Format(Cond_Pump_Power, "0.00E+0") & "  MWe"

OUTPUTTEXT12 = "Secondary Coolant Flowrate"

OUTPUTTEXT13 = Format(Sec_flowrate, "0.00E+0") & "  kg/s " & namecool2

OUTPUTTEXT14 = "Electrical Power Generation"

OUTPUTTEXT15 = Format(Turbine_Power, "0.00E+0") & "  MWe"

OUTPUTTEXT16 = ""

OUTPUTTEXT17 = ""

OUTPUTTEXT18 = ""

OUTPUTTEXT19 = ""

OUTPUTTEXT20 = ""

OUTPUTTEXT21 = ""

OUTPUTTEXT22 = ""

OUTPUTTEXT23 = ""

OUTPUTTEXT24 = ""

OUTPUTTEXT25 = ""

frmout.Show 1

End Sub

Sub MnuSodium_click ()

image13.Visible = True

image16.Visible = True

namecool1 = "Sodium"

sodium

image5.Picture = GroupPush3D4.PictureUp

image6.Picture = GroupPush3D13.PictureUp

End Sub

Sub MnuSpherical_click ()

INPUTTEXT1 = "INPUT Spherical Dimensions"

INPUTTEXT2 = "Set Reacor Radius"

inputtext3 = r & "  cm"

InputText4 = ""

inputtext5 = ""

InputText6 = ""

inputtext7 = ""

InputText8 = ""

inputtext9 = ""

InputText10 = ""

inputtext11 = ""

InputText12 = ""

inputtext13 = ""

FrmInput.Show 1

r = Val(inputtext3)

Volume = (4 / 3) * pi * ((Val(inputtext3)) ^ 3) / 1000000

namegeo = "Spherical   "

Buckling = (pi / Val(inputtext3)) ^ 2

image2.Picture = GroupPush3D17.PictureUp

image1.Visible = True

image17.Visible = True

image18.Visible = False

image19.Visible = False

End Sub

Sub MnuU233_click ()

namefuel = "U-233"

crossfiss = 514

crosscatf = 14

fcrossabs = 569

densityf = 19

Af = 233

thermalconduct = 14

image4.Picture = GroupPush3D7.PictureUp

End Sub

Sub MnuU235_click ()

namefuel = "U-235"

crossfiss = 582

crosscatf = 15

fcrossabs = 679

densityf = 19

Af = 235

thermalconduct = 14

image4.Picture = GroupPush3D8.PictureUp

End Sub

Sub MnuVariables_Click ()

INPUTTEXT1 = "INPUT Variables"

INPUTTEXT2 = "Maximum Coolant Temperature "

inputtext3 = maxtemp & "  degree K"

InputText4 = "Minimum Coolant Temperature"

inputtext5 = mintemp & "  degree K"

InputText6 = "Coolant Tube Diameter"

inputtext7 = tubedia & "  cm"

InputText8 = "Reactor Porosity"

inputtext9 = porosity

InputText10 = "Coolant Flowrate"

inputtext11 = flowrate & "  kg/s"

InputText12 = "   "

inputtext13 = "  "

FrmInput.Show 1

maxtemp = Val(inputtext3)

mintemp = Val(inputtext5)

tubedia = Val(inputtext7)

porosity = Val(inputtext9)

flowrate = Val(inputtext11)

End Sub

Sub MnuWater_click ()

namemod = "H2O"

diffus = 7.617

fermi = 31.36

densitym = 1

mcrossabs = .66

Am = 18

image3.Picture = GroupPush3D13.PictureUp

image12.Picture = GroupPush3D13.PictureUp

End Sub

Sub Timer1_Timer ()

Static PickBmp As Integer

image16.Move image16.Left + 50, image16.Top - 35

If PickBmp Then

image16.Picture = image15.Picture

Else

image16.Picture = image14.Picture

End If

image16.Move image16.Left - 50, image16.Top + 35

PickBmp = Not PickBmp

End Sub

///////////////////////////////

Sub Form_Load ()

decimal Powerth = 100

decimal crossfiss = 582;

decimal crossabs1 = 106;

decimal thermalconduct = 14;

decimal  crosscatf = 15;

decimal  fcrossabs = 679;

decimal  densityf = 19;

decimal  Af = 235;

decimal  diffus = 7.617;

decimal  fermi = 31.36;

decimal  densitym = 1;

decimal  mcrossabs = .66;

decimal  ccrossabs = .66;

decimal  Am = 18;

decimal  LX = 100;

decimal  LY = 100;

decimal  LZ = 100;

decimal  cr = 50;

decimal  r = 50;

decimal  cheight = 100;

decimal  Volume = (LX * LY * LZ) / (1000000);

decimal Buckling = ((pi / LX) ^ 2 + (pi / LY) ^ 2 + (pi / LZ) ^ 2);

decimal  porosity = .33;

decimal  tubedia = .4;

decimal  flowrate = 10000;

decimal  tubelength = 100;

decimal  Effic = 34;

decimal  numbertubes = 2600;

decimal   maxtemp = 530;

decimal   mintemp = 470;

decimal   FuelTemp = 877;

decimal   namefuel = "U-235";

decimal   namemod = "H2O";

string  namecool1 = "H2O";

string namecool2 = "H2O";

string namegeo = "Rectangular ";

decimal pressurec = 46 * safetyfactor;

decimal viscosity = .16;

decimal  Pri_Pressure = 100;

decimal Sec_Temperature(4) = 516;

decimal Sec_Pressure(4) = 34.9;

decimal Sec_Enthalpy(4) = 1205;

decimal Sec_Temperature(5) = 52;

decimal Sec_Pressure(5) = .1436;

decimal Sec_Enthalpy(5) = 850;

decimal Sec_Temperature(6) = 41;

decimal Sec_Pressure(6) = .0769;

decimal Sec_Enthalpy(6) = 68;

decimal Sec_Temperature(7) = 104;

decimal Sec_Pressure(7) = 34.9;

decimal Sec_Enthalpy(7) = 168;

tubearea = (pi * tubedia * tubelength * numbertubes);

heatflux = (Powerth * 1000000) / (tubearea);

Calculation_Phase

string Buffernamecool2 = "H2";

string Buffernamecool1 = "H2";

string Buffernamefuel = "Pu-241";

string  Buffernamemod = "H2O";

string  namecool1 = "H2O";

string  namecool2 = "H2O";

string  MnuWater_click;

Sub MnuAmericium242_click ()

namefuel = "Am-242"

crossfiss = 6000

crosscatf = 12

fcrossabs = 7700

densityf = 19

Af = 242

thermalconduct = 14

image4.Picture = GroupPush3D12.PictureUp

End Sub

Sub MnuBerylium_Click ()

namemod = "Be"

diffus = 441

fermi = 84.64

densitym = 1.85

mcrossabs = .01

Am = 9

image3.Picture = GroupPush3D3.PictureUp

image12.Picture = GroupPush3D3.PictureUp

End Sub

Sub MnuCarbon12_click ()

namemod = "C-12"

diffus = 4121

fermi = 350

densitym = 1.9

mcrossabs = .004

Am = 12

image3.Picture = GroupPush3D5.PictureUp

image12.Picture = GroupPush3D5.PictureUp

End Sub

Sub MnuCarbon13_CLICK ()

namemod = "C-13"

diffus = 4121

fermi = 350

densitym = 1.9

mcrossabs = .002

Am = 13

image3.Picture = GroupPush3D6.PictureUp

image12.Picture = GroupPush3D6.PictureUp

End Sub

Sub MnuCurium245_click ()

namefuel = "Cm-245"

crossfiss = 1700

crosscatf = 20

fcrossabs = 2000

densityf = 19

Af = 245

thermalconduct = 14

image4.Picture = GroupPush3D11.PictureUp

End Sub

cr = Val(inputtext3)

cheight = Val(inputtext5)

Volume = Val(inputtext5) * pi * ((Val(inputtext3)) ^ 2) / 1000000

namegeo = "Cylindrical "

Buckling = (2.405 / (Val(inputtext3))) ^ 2 + (pi / (Val(inputtext5))) ^ 2

Sub MnuDesignSummary_Click ()

outputtext1 = "DESIGN SUMMARY"

If namegeo = "Rectangular " Then

OUTPUTTEXT2 = "Length "

OUTPUTTEXT3 = LX & "  cm"

OUTPUTTEXT4 = "Height"

OUTPUTTEXT5 = LY & "  cm"

OUTPUTTEXT6 = "Width"

OUTPUTTEXT7 = LZ & "  cm"

End If

' Format(tubedia, "0.00E+00") & "  cm"

If namegeo = "Cylindrical " Then

OUTPUTTEXT2 = "Cylindrical Radius"

OUTPUTTEXT3 = cr & "  cm"

OUTPUTTEXT4 = "Height"

OUTPUTTEXT5 = cheight & "  cm"

OUTPUTTEXT6 = ""

OUTPUTTEXT7 = ""

End If

If namegeo = "Spherical   " Then

OUTPUTTEXT2 = "Spherical Radius"

OUTPUTTEXT3 = r & "  cm"

OUTPUTTEXT4 = ""

OUTPUTTEXT5 = ""

OUTPUTTEXT6 = ""

OUTPUTTEXT7 = ""

End If

Sub MnuDesignYourOwnFuel_Click ()

INPUTTEXT1 = "DESIGN YOUR OWN NUCLEAR FUEL INPUT"

INPUTTEXT2 = "Thermal Fission Cross Section"

inputtext3 = crossfiss & "  barns"

InputText4 = "Thermal Absorption Cross Section"

inputtext5 = fcrossabs & "  barns"

InputText6 = "Thermal Conductivity"

inputtext7 = thermalconduct & "  BTU/hr.ft.F"

InputText8 = "Density"

inputtext9 = densityf & " g/cc"

InputText10 = "Atomic Mass Number"

inputtext11 = Af

InputText12 = "Fuel Name"

inputtext13 = namefuel

FrmInput.Show 1

crossfiss = Val(inputtext3)

fcrossabs = Val(inputtext5)

thermalconduct = Val(inputtext7)

densityf = Val(inputtext9)

Af = Val(inputtext11)

namefuel = inputtext13

image4.Picture = GroupPush3D16.PictureUp

End Sub

Sub MnuDesignYourOwnMod_Click ()

INPUTTEXT1 = "DESIGN YOUR OWN MODERATOR"

INPUTTEXT2 = "Diffusion Length Squared"

inputtext3 = diffus & "  cm^-2"

InputText4 = "Fermi Age"

inputtext5 = fermi & "  cm^-2"

InputText6 = "Thermal Absorption Cross Section"

inputtext7 = mcrossabs & "  barns"

InputText8 = "Density"

inputtext9 = densitym & "  g/cc"

InputText10 = "Mass Number"

inputtext11 = Am

InputText12 = "Moderator Name"

inputtext13 = namemod

FrmInput.Show 1

diffus = Val(inputtext3)

fermi = Val(inputtext5)

mcrossabs = Val(inputtext7)

densitym = Val(inputtext9)

Am = Val(inputtext11)

namemod = inputtext13

image3.Picture = GroupPush3D15.PictureUp

image12.Picture = GroupPush3D15.PictureUp

End Sub

Sub MnuFileOpen_Click ()

On Error GoTo ErrHandler

CMDialog1.Filter = "All Files (*.*)|*.*|Text Files (*.txt)|*.txt|Batch Files (*.bat)|*.bat"

CMDialog1.FilterIndex = 2

CMDialog1.Action = 1

myfile = CMDialog1.Filename

Open myfile For Binary As #1

For i = 1 To 13

Get #1, , save(i)

Next i

Close #1

If namegeo = "Rectangular " Then

LX = Val(save(11))

LY = Val(save(12))

LZ = Val(save(13))

Volume = LX * LY * LZ / (1000000)

Buckling = ((pi / LX) ^ 2) + ((pi / LY) ^ 2) + ((pi / LZ) ^ 2)

End If

If namegeo = "Cylindrical " Then

cr = Val(save(11))

cheight = Val(save(12))

Volume = (pi * (cr * cr) * cheight) / (1000000)

tubelength = cr

Buckling = ((2.405 / cr) ^ 2 + (pi / cheight) ^ 2)

End If

If namegeo = "Spherical   " Then

r = Val(save(11))

Volume = (1.33 * pi * r * r * r) / (1000000)

tubelength = r

Buckling = (pi / r) ^ 2

End If

'End If

'If (lx > ly) And (lx > lz) Then

'tubelength = lx

'End If

' If (ly > lx) And (ly > lz) Then

'tubelength = ly

'  End If

'If (lz > lx) And (lz > ly) Then

' tubelength = lz

If namefuel = "U-233       " Then

MnuU233_click

End If

If namefuel = "U-235       " Then

MnuU235_click

End If

If namefuel = "Pu-239      " Then

MnuPlutonium239_click

End If

If namefuel = "Pu-241      " Then

MnuPlutonium241_click

End If

If namefuel = "Am-242      " Then

MnuAmericium242_click

End If

If namefuel = "Cm-24       " Then

MnuCurium245_click

End If

If namemod = "H2O         " Then

MnuWater_click

End If

If namemod = "D2O         " Then

MnuHeavyWater_CLICK

End If

If namemod = "Be          " Then

MnuBerylium_Click

End If

If namemod = "C-12        " Then

MnuCarbon12_click

End If

If namemod = "C-13        " Then

MnuCarbon13_CLICK

End If

If namecool1 = "H2O         " Then

MnuWater_click

End If

If namecool1 = "H2          " Then

MnuHydrogen_click

End If

If namecool1 = "He          " Then

MnuHelium_click

End If

If namecool1 = "SODIUM      " Then

MnuSodium_click

End If

Calculation_Phase

End Sub

Sub MnuFileSave_Click ()

save(1) = namefuel

save(2) = namemod

save(3) = namecool1

save(4) = namecool2

save(5) = namegeo

save(6) = Str(maxtemp)

save(7) = Str(mintemp)

save(8) = Str(tubedia)

save(9) = Str(porosity)

save(10) = Str(flowrate)

If namegeo = "Rectangular " Then

save(11) = Str(LX)

save(12) = Str(LY)

save(13) = Str(LZ)

End If

If namegeo = "Cylindrical " Then

save(11) = Str(cr)

save(12) = Str(cheight)

save(13) = "            "

End If

If namegeo = "Spherical   " Then

save(11) = Str(r)

save(12) = "            "

save(13) = "            "

End If

On Error GoTo ErrHandler1

CMDialog1.Filter = "All Files (*.*)|*.*|Text Files (*.txt)|*.txt|Batch Files (*.bat)|*.bat"

CMDialog1.FilterIndex = 2

CMDialog1.Action = 2

myfile = CMDialog1.Filename

Open myfile For Binary As #1

For i = 1 To 13

Put #1, , save(i)

Next i

Close #1

ErrHandler1:

Exit Sub

End Sub

Sub MnuHeavyWater_CLICK ()

namemod = "D2O"

diffus = 10000

fermi = 121

densitym = 1.1

mcrossabs = .002

Am = 20

End Sub

Sub MnuHelium_click ()

image13.Visible = True

image16.Visible = True

namecool1 = "Helium"

namecool2 = "H2O"

helium

Sub MnuHydrogen_click ()

image13.Visible = False

image16.Visible = False

namecool1 = "Hydrogen"

namecool2 = "Hydrogen"

hydrogen

End Sub

Sub MnuPlutonium239_click ()

namefuel = "Pu-239"

crossfiss = 760

crosscatf = 11

fcrossabs = 1030

densityf = 19

Af = 239

thermalconduct = 14

End Sub

Sub MnuPlutonium241_click ()

namefuel = "Pu-241"

crossfiss = 988

crosscatf = 10

fcrossabs = 1384

densityf = 19

Af = 241

thermalconduct = 14

End Sub

Sub MnuPrimaryCoolant_Click ()

Calculation_Phase

outputtext1 = "SUMMARY OF PRIMARY COOLANT RESULTS"

OUTPUTTEXT2 = "Reactor Porosity or Void Fraction"

OUTPUTTEXT3 = Format(porosity, "0.00E+00")

OUTPUTTEXT4 = "Coolant Tube Diameter"

OUTPUTTEXT5 = Format(tubedia, "0.00E+00") & "  cm"

OUTPUTTEXT6 = "Number Of Coolant Channels"

OUTPUTTEXT7 = Format(numbertubes, "0.00E+00")

OUTPUTTEXT8 = "Primary Coolant Pressure"

OUTPUTTEXT9 = Format(Pri_Pressure, "0.00E+00") & "  atmos"

OUTPUTTEXT10 = "Primary Coolant Flowrate"

OUTPUTTEXT11 = Format(flowrate, "0.00E+00") & "  kg/s"

OUTPUTTEXT12 = "Primary Coolant Velocity"

OUTPUTTEXT13 = Format(velocity, "0.00E+00") & "  m/s"

OUTPUTTEXT14 = "Reactor Pressure Drop"

OUTPUTTEXT15 = Format(Rx_Pressure_Drop, "0.00E+00") & "  atmos"

OUTPUTTEXT16 = "Reactor Pump Power"

OUTPUTTEXT17 = Format(Pump_Power, "0.00E+00") & "  MWe"

OUTPUTTEXT18 = "Coolant Mass within Reactor"

OUTPUTTEXT19 = Format(massc, "0.00E+00") & "  Mg " & namecool1

OUTPUTTEXT20 = "Max Primary Coolant Temperature"

OUTPUTTEXT21 = Format(maxtemp, "0.00E+00") & "  degree K"

OUTPUTTEXT22 = "Differential Coolant Temperature"

OUTPUTTEXT23 = Format(DeltaT, "0.00E+00") & "  degree K"

OUTPUTTEXT24 = ""

OUTPUTTEXT25 = ""

frmout.Show 1

End Sub

Sub MnuPrint_Click ()

On Error GoTo ErrHandler2

CMDialog1.Filter = "All Files (*.*)|*.*|Text Files (*.txt)|*.txt|Batch Files (*.bat)|*.bat"

CMDialog1.FilterIndex = 2

CMDialog1.Action = 5

Sub MnuReactorResults_Click ()

Calculation_Phase

outputtext1 = "SUMMARY OF NUCLEAR REACTOR DESIGN RESULTS" & Chr(13) & Chr(10) & "hhhhhh"

OUTPUTTEXT2 = "Volume of Reactor"

OUTPUTTEXT3 = Format(Volume, "0.00E+00") & "  m^3 "

OUTPUTTEXT4 = "Reactor Geometric Buckling"

OUTPUTTEXT5 = Format(Buckling, "0.00E+00") & "  cm^-2"

OUTPUTTEXT6 = "Moderator-to-Fuel Ratio"

OUTPUTTEXT7 = Format(ratio, "0.00E+00") & "  "

OUTPUTTEXT8 = "Fuel Number Density"

OUTPUTTEXT9 = Format(nf, "0.00E+00") & "  atoms/cc"

OUTPUTTEXT10 = "Moderator Mass"

OUTPUTTEXT11 = Format(massm, "0.00E+00") & "  MG " & namemod

OUTPUTTEXT12 = "Critical Mass"

OUTPUTTEXT13 = Format(critmass, "0.00E+00") & "  kg " & namefuel

OUTPUTTEXT14 = "Annual Fuel Burnup"

OUTPUTTEXT15 = Format(burnup, "0.00E+00") & "  Mg/yr " & namefuel

OUTPUTTEXT16 = "Specific Power"

OUTPUTTEXT17 = Format(specificpower, "0.00E+00") & " kw/gram " & namefuel

OUTPUTTEXT18 = "Avg. Heat Flux"

OUTPUTTEXT19 = Format(heatflux, "0.00E+00") & "  watts/cm^2"

OUTPUTTEXT20 = "Maximum Fuel Temperature"

OUTPUTTEXT21 = Format(FuelTemp, "0.00E+00") & "  degree K"

OUTPUTTEXT22 = "Avg. Thermal Neutron Flux"

OUTPUTTEXT23 = Format(N_Flux, "0.00E+00") & " /sec-cm^2"

OUTPUTTEXT24 = "Thermal Power Produced"

OUTPUTTEXT25 = Format(Powerth, "0.00E+00") & "  MWth "

frmout.Show 1

End Sub

Sub MnuRectangular_click ()

INPUTTEXT1 = "INPUT"

INPUTTEXT2 = "Set Reacor Length"

inputtext3 = LX & "  cm"

InputText4 = "Set Reactor Width"

inputtext5 = LY & "  cm"

InputText6 = "Set Reactor Height"

inputtext7 = LZ & "  cm"

FrmInput.Show 1

LX = Val(inputtext3)

LY = Val(inputtext5)

LZ = Val(inputtext7)

Volume = (Val(inputtext3) * Val(inputtext5) * Val(inputtext7) / 1000000)

namegeo = "Rectangular "

tubelength = Val(inputtext3)

Buckling = ((pi / Val(inputtext3)) ^ 2 + (pi / Val(inputtext5)) ^ 2 + (pi / Val(inputtext7)) ^ 2)

image2.Picture = GroupPush3D19.PictureUp

End Sub

Sub MnuSecCoolant_Click ()

Calculation_Phase

outputtext1 = "SUMMARY OF SECONDARY COOLANT RESULTS"

OUTPUTTEXT2 = "Steam Temerature "

OUTPUTTEXT3 = Format(Sec_Temperature(4), "0.00E+0") & "  degree K"

OUTPUTTEXT4 = "Steam generator Pressure(4) "

OUTPUTTEXT5 = Format(Sec_Pressure(4), "0.00E+0") & "  atmos"

OUTPUTTEXT6 = "Turbine Efficiency"

OUTPUTTEXT7 = Format(80, "0.00E+0") & "  %"

OUTPUTTEXT8 = "Secondary Efficiency "

OUTPUTTEXT9 = Format(Effic, "0.00E+0") & "  %"

OUTPUTTEXT10 = "Condensate Pump Power"

OUTPUTTEXT11 = Format(Cond_Pump_Power, "0.00E+0") & "  MWe"

OUTPUTTEXT12 = "Secondary Coolant Flowrate"

OUTPUTTEXT13 = Format(Sec_flowrate, "0.00E+0") & "  kg/s " & namecool2

OUTPUTTEXT14 = "Electrical Power Generation"

OUTPUTTEXT15 = Format(Turbine_Power, "0.00E+0") & "  MWe"

frmout.Show 1

End Sub

Sub MnuSodium_click ()

image13.Visible = True

image16.Visible = True

namecool1 = "Sodium"

sodium

image5.Picture = GroupPush3D4.PictureUp

image6.Picture = GroupPush3D13.PictureUp

End Sub

Sub MnuSpherical_click ()

INPUTTEXT1 = "INPUT Spherical Dimensions"

INPUTTEXT2 = "Set Reacor Radius"

inputtext3 = r & "  cm"

FrmInput.Show 1

r = Val(inputtext3)

Volume = (4 / 3) * pi * ((Val(inputtext3)) ^ 3) / 1000000

namegeo = "Spherical   "

Buckling = (pi / Val(inputtext3)) ^ 2

image2.Picture = GroupPush3D17.PictureUp

image1.Visible = True

image17.Visible = True

image18.Visible = False

image19.Visible = False

End Sub

Sub MnuU233_click ()

namefuel = "U-233"

crossfiss = 514

crosscatf = 14

fcrossabs = 569

densityf = 19

Af = 233

thermalconduct = 14

image4.Picture = GroupPush3D7.PictureUp

End Sub

Sub MnuU235_click ()

namefuel = "U-235"

crossfiss = 582

crosscatf = 15

fcrossabs = 679

densityf = 19

Af = 235

thermalconduct = 14

End Sub

Sub MnuVariables_Click ()

INPUTTEXT1 = "INPUT Variables"

INPUTTEXT2 = "Maximum Coolant Temperature "

inputtext3 = maxtemp & "  degree K"

InputText4 = "Minimum Coolant Temperature"

inputtext5 = mintemp & "  degree K"

InputText6 = "Coolant Tube Diameter"

inputtext7 = tubedia & "  cm"

InputText8 = "Reactor Porosity"

inputtext9 = porosity

InputText10 = "Coolant Flowrate"

inputtext11 = flowrate & "  kg/s"

InputText12 = "   "

inputtext13 = "  "

End Sub

Sub MnuWater_click ()

namemod = "H2O"

diffus = 7.617

fermi = 31.36

densitym = 1

mcrossabs = .66

Am = 18

End Sub